Summary:

Overview

The MultiADC_Div VHDL project is designed to collect real-time data from two independent external ADCs interfaced with the Arty A7-35T FPGA board. Each ADC acquires data from a different analog source. The digitized signals are processed by the FPGA and then streamed to a host PC for real-time data visualization. The project implements VHDL-based control and communication logic, and uses UART for data transmission.

Objective

- Interface two independent ADC modules with the FPGA using SPI protocol.

- Implement a FIFO buffer to store acquired samples.

- Control data flow using UART and AXI4 stream interfaces.

- Stream real-time data from the FPGA to a PC monitor for acquisition analysis.

FPGA Target

- Board: Digilent Arty A7-35T

- Family: Xilinx Artix-7

- Tool Used: Xilinx Vivado (simulation and synthesis)

Block diagram of data flow:

=> ADC Read Modules fetch data via SPI from ADC1 and ADC2. Data from each ADC is pushed into a generic FIFO. A UART interface reads FIFO data and sends it to a PC. The Top Module synchronizes all submodules.

Architecture:

- TOP Module: this module coordinates the full design. It: Instantiates the UartCommand module for PC communication, Interfaces with ADC drivers, Controls read/write operations for continuous streaming.

- ADC Control (Control_process): This process alternate between ADC1 and ADC2, reading from one ADC at a time:

- When data_ready is high, it checks which ADC is active using stateActive.

- Captures DataTime, DeviceID, DataType, and TransmitData.

Asserts readActive to trigger transmission.

- Deglitching Inputs (deglitch):

- Synchronizes asynchronous inputs like reset and UART RX to the system clock to prevent metastability.

- Timestamp Generator (timestampProcess):

- Generates a time counter (TimeCounter) using a prescaler (ReduceSpeed) to scale down the 100 MHz clock to 1 Hz. The counter is used to timestamp ADC data.

- Receiving Process (ReceiveProcess): Handles USB UART RX data:

- Decodes commands (x"31", x"32", x"33").

 Routes data to transmit FIFOs for each path.

 Dual channel support with toggling signals (sendLogic, sendLogic1).

- Transmit Process (transmitProcess): Sequentially transmits the data frame through UART:

- Waits for readActive to start.

 Sends fields like DataType, DataTime, DeviceID, TransmitData byte-by-byte.

 Ends with a newline character.

- UartCommand Module: This module instantiates the uart and GENERIC_FIFO modules and create a loopback process:

Reset : Clears all control and data signals on reset.

UART Receive → FIFO Store: If uart_data_out_stb = '1' and FIFO is not full, the received UART byte is stored in FIFO.

UART Transmit Acknowledge: Clears the uart_data_in_stb flag when transmission is acknowledged (uart_data_in_ack = '1').

FIFO Read → UART Send: If FIFO is not empty and UART is ready, a byte is read from FIFO and passed to UART for transmission.

- FIFO Module:

1. Reset: On reset, write_pointer and read_pointer are set to 0, fifo_memory is cleared, fifo_empty is '1', and fifo_full is '0'.

2. Write Operation: When write_en = '1' and the FIFO is not full, data is written to the memory at write_pointer, and write_pointer advances. If write_pointer reaches FIFO_DEPTH - 1, it wraps around to 0.

3. Read Operation: When read_en = '1' and the FIFO is not empty, read_pointer advances, and the data at read_pointer is output via read_data. If read_pointer reaches FIFO_DEPTH - 1, it wraps around to 0.

4. Status Updates: The FIFO_FLAGS process continuously monitors write_pointer and read_pointer to update full, empty, and level. The FIFO is considered full when level = FIFO_DEPTH - 1 to avoid ambiguity with the empty state (both pointers equal).

- ADC module: The adc_read module implements a serial communication protocol to read 16-bit data from an ADC:

• Idle State: When startConv = '0', the module waits with cs = '1', data_ready = '0', and count = 16.      

• Start Conversion: When startConv = '1', cs_int is set to '0', activating the ADC. The module counts down from 16 to 0 over 16 clock cycles, shifting in one bit of sdata per falling edge into shift_reg.

• Data Completion: When count reaches -1, the 16-bit data is complete in shift_reg. cs_int is set to '1', data_ready is set to '1', and out_data is assigned shift_reg. count is set to 17 to allow a brief pause before the next conversion.

• Reset: A reset clears all signals and returns the module to the idle state.

PMOD\PIN Configuration:

- Pmod Header JA

Signal Name 		Pin		 Description

sclk2 			D13		 Serial Clock 2

sdata2			 B18 		Serial Data 2

cs2 			A18 		Chip Select 2

- Pmod Header JB

Signal Name 		Pin		 Description

tx_1 			E15 		UART Transmit 1

rx_1 			E16 		UART Receive 1

rx_2 			J17		 UART Receive 2

tx_2 			J18 		UART Transmit 2

- Pmod Header JC

Signal Name		 Pin		 Description

sclk1 			U14		 Serial Clock 1

sdata1 		V14 		Serial Data 1

cs1 			T13		 Chip Select 1

- USB-UART Interface (Onboard)

Signal Name		 Pin		 Description

usb_rs232_txd 	D10 		Transmit from FPGA to PC

usb_rs232_rxd 	A9 		Receive from PC to FPGA

Reference:

Digilent Arty A7-35T Reference Manual

Xilinx Vivado Design Suite Documentation

UART and SPI Communication Protocols

VHDL 2008 Language Reference